KNOWLEDGE BASED FRAMEWORK FOR LOCALIZATION                  OF RETINAL LANDMARKS FROM DIABETIC                   RETINOPATHY (DR) IMAGES

ABSTRACT

Diabetes is a disease which occurs when the pancreas does not secrete enough insulin or the body is unable to process it properly. The Diabetic Retinopathy (DR) is a complication of diabetes that is caused by changes in the blood vessels of retina. The symptoms can distort or blur the patient’s vision and are the main causes of blindness. One of the difficulties in this illness is that the patient with diabetes mellitus requires a continuous screening for early detection. We propose an algorithm for the detection of retinal landmarks (optic nerve head or optic disc, macula, and vasculature) based on optic cup location and anatomical structural details from diabetic retinopathy (DR) images of both left and right eye. Our algorithm uses color fundus images obtained from mydriatic camera. This system intends to help the ophthalmologists not only in DR screening process but any other eye related abnormality which is based on retinal photography.

EXISTING SYSTEM

The existing system presents a method for automated segmentation of the vasculature in retinal images. The method produces segmentations by classifying each image pixel as vessel or non-vessel, based on the pixel's feature vector. Feature vectors are composed of the pixel's intensity and two-dimensional Gabor wavelet transform responses taken at multiple scales. The Gabor wavelet is capable of tuning to specific frequencies, thus allowing noise filtering and vessel enhancement in a single step. It uses a Bayesian classifier with class-conditional probability density functions (likelihoods) described as Gaussian mixtures, yielding a fast classification, while being able to model complex decision surfaces. The probability distributions are estimated based on a training set of labeled pixels obtained from manual segmentations.

DISADVANTAGES:

- Challenge of Gabor technique is it has high redundancy of features

- Low accuracy

PROPOSED SYSTEM

Diabetic Retinopathy (DR) is a complication of diabetes that is caused by changes in the blood vessels of retina. This Sytem proposes an algorithm for the detection of retinal landmarks based on optic cup location and anatomical structural details from diabetic retinopathy (DR) images of both left and right eye. The algorithm will proceed through four main steps: Color image pre-processing to enhance and remove noise from the image. The optic nerve head is located using a fact that the optic cup is the brightest region in the optic nerve head. Detection of macula Detection of vasculature by used logical AND operation on two images, one being a thresholded image and another being an edge detected image. The thrsholding is done on an adaptive histogram equalized image. Edge detection is done using canny edge detector. Proposing algorithm will be tested on both normal and DR images.

ADVANTAGES

- High Sensitivity, specificity  and predictive value

- adaptive histogram equalization improves the local contrast and enhances thresholding

SYSTEM REQUIREMENT:

HARDWARE REQUIREMENTS:

Processor			:	Intel Core i3 Processor

Speed				:   	2.5 GHz

RAM				:   	2GB (min)

Hard Disk			:	500MB

Key Board			:   	Standard Windows Keyboard

Mouse			:    	Two or Three Button Mouse

Monitor			:    	LCD

SOFTWARE CONFIGURATION:

Operating System		:	Windows 7 / 10

Environment		:	MATLAB

Matlab			:	Version 18a

REFERENCES

[1] M.Lalonde, M.Beaulieu and L.Gagnon, “Fast and robust optic disc detection using pyramidal decomposition and Hausdroffbased template matching”, IEEE Transactions on Medical Imaging, Vol.23, Issue 11, pp.1193-1200, November, 2001.

[2] J.Lowell ,A.Hunter,D.Steel, A.Basu,R.Ryder and E. Fletcher, “Optic Nerve Head Segmentation”, IEEE Transactions on Medical Imaging, Vol.23, No.2, pp.256-264, February 2004.

[3] Huiqi Li and Opas Chutatape, “Fundus image features extraction”, in Proceedings of the Annual IEEE International Conference of EMBS, Vol.4, pp. 3071-3073, July 2000.

[4] H.Li and O.Chutatape, “A model based approach for automated feature extraction in fundus images”, in Proceedings of the 9th International Conference on Computer Vision, pp. 3941-399, October 2003.

[5] Mira park, Jesse S, “Locating the optic disc in retinal images”, in Proceedings of the International Conference on Computer Graphics, Imaging and Visualization, pp.141-145, July 2006.