GRADIENT HISTOGRAM ESTIMATION AND PRESERVATION FOR TEXTURE ENHANCED IMAGE DENOISING

ABSTRACT

Natural image statistics plays an important role in image denoising, and various natural image priors, including gradient-based, sparse representation-based, and nonlocal self-similarity-based ones, have been widely studied and exploited for noise removal. In spite of the great success of many denoising algorithms, they tend to smooth the fine scale image textures when removing noise, degrading the image visual quality. To address this problem, in this paper, we propose a texture enhanced image denoising method by enforcing the gradient histogram of the denoised image to be close to a reference gradient histogram of the original image. Given the reference gradient histogram, a novel gradient histogram preservation (GHP) algorithm is developed to enhance the texture structures while removing noise. Two region-based variants of GHP are proposed for the denoising of images consisting of regions with different textures. An algorithm is also developed to effectively estimate the reference gradient histogram from the noisy observation of the unknown image. Our experimental results demonstrate that the proposed GHP algorithm can well preserve the texture appearance in the denoised images, making them look more natural.

EXISTING SYSTEM

Sparse representation models code an image patch as a linear combination of a few atoms chosen out from an over-complete dictionary, and they have shown promising results in various image restoration applications. However, due to the degradation of the observed image (e.g., noisy, blurred, and/or down-sampled), the sparse representations by conventional models may not be accurate enough for a faithful reconstruction of the original image. To improve the performance of sparse representation-based image restoration, in this paper the concept of sparse coding noise is introduced, and the goal of image restoration turns to how to suppress the sparse coding noise. To this end, we exploit the image nonlocal self-similarity to obtain good estimates of the sparse coding coefficients of the original image, and then centralize the sparse coding coefficients of the observed image to those estimates. The so-called nonlocally centralized sparse representation (NCSR) model is as simple as the standard sparse representation model, while our extensive experiments on various types of image restoration problems, including denoising, deblurring and super-resolution, validate the generality and state-of-the-art performance of the proposed NCSR algorithm.

DISADVANTAGES

- They degrade image details and the edges of the image.

- It is computationally expensive.

PROPOSED SYSTEM

In this proposed system, we presented a novel gradient histogram preservation (GHP) model for texture-enhanced image denoising, and further introduce two region-based GHP variants, i.e., B-GHP and S-GHP. A simple but theoretically solid model and the associated algorithm were presented to estimate the reference gradient histogram from the noisy image, and an efficient iterative histogram specification algorithm was developed to implement the GHP model. By pushing the gradient histogram of the denoised image toward the reference histogram, GHP achieves promising results in enhancing the texture structure while removing random noise. The experimental results demonstrated the effectiveness of GHP in texture enhanced image denoising. GHP leads to similar PSNR/SSIM measures to the state-of-the-art denoising methods such as SAPCABM3D, LSSC and NCSR; however, it leads to more natural and visually pleasant denoising results by better preserving the image texture areas. Most of the state-of-the-art denoising algorithms are based on the local sparsity and nonlocal selfsimilarity priors of natural images. Unlike them, the gradient histogram used in our GHP method is a kind of global prior, which is adaptively estimated from the given noisy image. One limitation of GHP is that it cannot be directly applied to non-additive noise removal, such as multiplicative Poisson noise and signal-dependent noise.

ADVANTAGES

- It reduces noise in flat regions and preserves edges to a higher extend

- Moe robust and preserve fine structure to some extend

SYSTEM REQUIREMENT:

HARDWARE REQUIREMENTS:

Processor 		: 	Intel

Ram			:	2 GB (Minimum)

Monitor		:	15” COLOR

Hard Disk		:	500 GB

Keyboard 		:	STANDARD 102 KEYS

Mouse		:	3 BUTTONS

SOFTWARE CONFIGURATION:

Operating System		:	Windows 7 / 10

Environment		:	MATLAB

Matlab		 	:	Version 18a

REFERENCES

- R. Fergus, B. Singh, A. Hertzmann, S. Roweis, and W. T. Freeman, “Removing camera shake from a single photograph,” in Proc. ACM SIGGRAPH, 2006, pp. 787–794.

- A. Levin, R. Fergus, F. Durand, and W. T. Freeman, “Image and depth from a conventional camera with a coded aperture,” in Proc. ACM SIGGRAPH, 2007.

- D. Krishnan and R. Fergus, “Fast image deconvolution using hyperLaplacian priors,” in Proc. NIPS, 2009, pp. 1033–1041.

- L. Rudin, S. Osher, and E. Fatemi, “Nonlinear total variation based noise removal algorithms,” Phys. D, vol. 60, nos. 1–4, pp. 259–268, Nov. 1992.

- M. Wainwright and S. Simoncelli, “Scale mixtures of Gaussians and the statistics of natural images,” in Proc. NIPS, vol. 12. 1999, pp. 855–861.